# BNSAT-R RESEARCH PROTOCOL v1.0

## Bitcoin Node Security & Resilience Research

### Proposed Experimental Framework for the Bitcoin Node Resilience Benchmark

Research Programme: BNSAT-R
Proposed Experimental Project: BNRB - Bitcoin Node Resilience Benchmark
Lead Organisation: UBRO CLASSIFIEDS LIMITED
Location: Kampala, Uganda
Protocol Version: 1.0
Status: Proposed for Technical Review
Date: July 2026

# 1. Executive Summary

This document defines the proposed research methodology for the BNRB - Bitcoin Node Resilience Benchmark - an experimental research project under the BNSAT-R programme of UBRO CLASSIFIEDS LIMITED.

The purpose of BNRB is to investigate whether the operational resilience of individual Bitcoin full-node deployments can be measured systematically and reproducibly under controlled infrastructure disruptions and resource constraints.

The research does not seek to modify Bitcoin’s consensus rules, interfere with the public Bitcoin network or evaluate the security of Bitcoin’s protocol itself.

Instead, it focuses on the operational layer surrounding an individual full-node deployment.

The proposed research investigates the following broad question:

How do Bitcoin full-node deployments behave when exposed to controlled disruptions affecting power availability, network connectivity, storage capacity and computing resources, and how consistently can their recovery behaviour be measured?

The research will establish a controlled laboratory environment containing multiple experimental node profiles.

Experiments will compare a baseline node with nodes operating under defined resource constraints and controlled disruptions.

Potential metrics include:

- Recovery time;

- Recovery success;

- Synchronisation recovery;

- Availability;

- Data integrity;

- Resource consumption;

- Human intervention;

- Recovery complexity.

The protocol is designed to be reproducible, transparent and safe.

No experiments will be conducted against unauthorised third-party systems or public Bitcoin infrastructure.

The initial protocol is a research proposal and must undergo technical review before implementation.

# 2. Research Purpose

The purpose of BNRB is to determine whether a reproducible methodology can be developed for measuring the operational resilience of Bitcoin full-node deployments.

The project has three primary goals.

## Goal 1 - Understand

Determine how individual nodes behave under controlled operational disruptions.

## Goal 2 - Measure

Develop meaningful and reproducible resilience metrics.

## Goal 3 - Contribute

If the methodology proves useful, publish the research findings and, where appropriate, release open-source tools and documentation.

# 3. Research Question

The primary research question is:

Can the operational resilience of individual Bitcoin full-node deployments be measured systematically under controlled infrastructure disruptions and resource constraints?

# 4. Secondary Research Questions

### RQ1

What happens to a Bitcoin full node following an unexpected shutdown?

### RQ2

How does network interruption affect node recovery?

### RQ3

How does intermittent connectivity affect synchronisation recovery?

### RQ4

How do storage constraints affect operational resilience?

### RQ5

How do CPU and memory constraints affect recovery behaviour?

### RQ6

How much human intervention is required following different failure scenarios?

### RQ7

Can recovery behaviour be measured consistently across different hardware profiles?

### RQ8

Can resilience metrics be standardised sufficiently to enable meaningful comparisons?

# 5. Research Hypothesis

The initial hypothesis is:

Bitcoin full-node deployments exhibit measurable differences in operational resilience and recovery behaviour when exposed to different classes of controlled infrastructure disruptions and resource constraints.

A secondary hypothesis is:

A standardised experimental methodology can identify and quantify meaningful differences in recovery behaviour across node configurations.

These hypotheses may be supported, modified or rejected.

The research will not assume that the hypotheses are correct.

# 6. Research Scope

The research focuses on the operational resilience of individual Bitcoin full-node deployments.

The research includes:

- Bitcoin Core full nodes;

- Node hardware;

- Node operating systems;

- Storage;

- Memory;

- CPU;

- Network connectivity;

- Power availability;

- Recovery procedures;

- Synchronisation behaviour.

The research does not initially focus on:

- Bitcoin price;

- Trading;

- Wallet security;

- Exchange security;

- Mining profitability;

- Consensus-rule changes;

- Bitcoin governance;

- Attacks against the public Bitcoin network.

# 7. Experimental Environment

The initial research laboratory should be established in Kampala, Uganda.

The laboratory should be physically and logically isolated from production systems.

The proposed environment consists of:

INTERNET
                       │
                 CONTROLLED ROUTER
                       │
                 TEST NETWORK
                       │
        ┌──────────────┼──────────────┐
        │              │              │
        ▼              ▼              ▼
    NODE A          NODE B          NODE C
    BASELINE       CONSTRAINED     VARIABLE
        │              │              │
        └──────────────┼──────────────┘
                       │
                       ▼
                 DATA COLLECTOR
                       │
                       ▼
                 ANALYSIS SERVER

# 8. Proposed Hardware Profiles

The research should initially use at least three node profiles.

## Profile A - Baseline Node

Purpose:

Establish reference behaviour.

Suggested characteristics:

- Modern x86-64 system;

- 4 or more CPU cores;

- 16 GB RAM;

- SSD storage;

- Reliable power;

- Stable Internet connection.

The exact hardware specification should be documented.

## Profile B - Resource-Constrained Node

Purpose:

Represent a lower-resource deployment.

Suggested characteristics:

- Lower CPU capacity;

- 4–8 GB RAM;

- SSD storage;

- Limited bandwidth;

- Controlled resource limits.

The objective is not to represent every low-resource environment.

The objective is to create a reproducible constrained profile.

## Profile C - Experimental Variable Node

Purpose:

Allow controlled manipulation of one variable at a time.

Examples:

- Reduced memory;

- Reduced CPU;

- Reduced bandwidth;

- Increased storage pressure.

This profile allows individual variables to be isolated.

# 9. Operating System

The initial experiments should use a stable, documented Linux distribution.

The exact operating-system version should be recorded as part of the experimental metadata.

The system configuration should include:

- OS version;

- Kernel version;

- CPU architecture;

- CPU model;

- RAM;

- Storage type;

- Storage capacity;

- Filesystem;

- Network interface;

- Network configuration.

The laboratory should avoid unnecessary software variation.

# 10. Bitcoin Core Version

The research should use a current supported Bitcoin Core release at the time experiments begin.

The exact version must be recorded.

The protocol should not assume that a particular version remains current throughout the research programme.

For reproducibility, each experiment must record:

- Bitcoin Core version;

- Commit or release identifier where practical;

- Configuration parameters;

- Build method;

- Operating system;

- Hardware profile.

The current research review indicates that Bitcoin Core 31.0 is a relevant current release in the 2026 environment, but the final experimental version should be selected immediately before laboratory deployment and recorded in the experiment metadata.

# 11. Node Configuration

Each experimental node must have a documented configuration.

The following should be recorded:

- Bitcoin Core version;

- Configuration file;

- Data directory;

- Pruning status;

- Indexing configuration;

- Database cache;

- Mempool configuration;

- Maximum connections;

- Network configuration;

- Peer configuration;

- Logging configuration.

No configuration should be changed during an experiment unless the change is itself the experimental variable.

# 12. Experimental Variables

The study will classify variables into four categories.

## Independent Variables

Variables intentionally changed by researchers.

Examples:

- Power availability;

- Network availability;

- Bandwidth;

- CPU allocation;

- Memory allocation;

- Storage availability.

## Dependent Variables

Variables measured as outcomes.

Examples:

- Recovery time;

- Synchronisation state;

- Availability;

- Resource utilisation;

- Human intervention.

## Control Variables

Variables kept constant.

Examples:

- Bitcoin Core version;

- Hardware;

- OS;

- Test duration;

- Experiment procedure.

## Confounding Variables

Variables that could unintentionally affect results.

Examples:

- Background processes;

- External Internet congestion;

- Hardware thermal throttling;

- Unexpected system updates;

- Network routing changes.

Confounding variables must be documented where identified.

# 13. Baseline Establishment

Before any disruption experiment, each node must operate normally for a defined baseline period.

During the baseline period, the following should be recorded:

- CPU utilisation;

- Memory utilisation;

- Disk utilisation;

- Disk I/O;

- Network traffic;

- Peer count;

- Blockchain height;

- Synchronisation state;

- Node uptime.

The baseline establishes the expected operating profile.

No disruption experiment should begin until baseline behaviour is sufficiently stable.

# 14. Experiment Categories

The initial research will investigate five primary categories.

## Category A - Power Disruption

## Category B - Network Disruption

## Category C - Storage Constraint

## Category D - CPU and Memory Constraint

## Category E - Combined Constraints

Combined scenarios should only be introduced after individual variables have been tested.

This prevents early experiments from becoming difficult to interpret.

# 15. Experiment A - Graceful Shutdown

### Objective

Establish normal recovery following an intentional shutdown.

### Procedure

- Establish baseline.

- Record node state.

- Gracefully shut down Bitcoin Core.

- Record shutdown time.

- Restart the node.

- Monitor recovery.

- Record synchronisation status.

### Measurements

- Shutdown duration;

- Restart time;

- Recovery time;

- Synchronisation status;

- Manual intervention.

This serves as a control experiment.

# 16. Experiment B - Unexpected Shutdown

### Objective

Measure recovery following an unexpected process or system interruption.

### Procedure

- Establish baseline.

- Record node state.

- Simulate an unexpected shutdown within the controlled laboratory.

- Restart the system.

- Monitor Bitcoin Core.

- Record recovery behaviour.

### Measurements

- Startup time;

- Recovery time;

- Data integrity;

- Synchronisation state;

- Manual intervention.

# 17. Experiment C - Network Interruption

### Objective

Measure node behaviour during loss of network connectivity.

### Scenarios

- Short interruption;

- Medium interruption;

- Extended interruption.

### Measurements

- Peer count;

- Connection recovery;

- Synchronisation state;

- Time to normal operation.

# 18. Experiment D - Intermittent Connectivity

### Objective

Investigate the effect of unstable connectivity.

The experiment may simulate:

- Repeated connection loss;

- Short connection windows;

- Variable latency;

- Packet loss;

- Bandwidth limitation.

The exact parameters must be selected during pilot testing.

### Measurements

- Synchronisation progress;

- Peer stability;

- Recovery time;

- Data transfer;

- Resource utilisation.

# 19. Experiment E - Storage Pressure

### Objective

Investigate the effect of limited available storage.

The experiment should use controlled test environments.

The study must avoid uncontrolled depletion of production storage.

### Measurements

- Disk utilisation;

- Node behaviour;

- Error conditions;

- Recovery;

- Synchronisation.

# 20. Experiment F - CPU Constraint

### Objective

Determine whether CPU limitations affect operational resilience.

CPU resources may be intentionally constrained using controlled system-level mechanisms.

### Measurements

- CPU utilisation;

- Processing time;

- Recovery time;

- Synchronisation progress;

- System stability.

# 21. Experiment G - Memory Constraint

### Objective

Investigate whether reduced memory availability affects node recovery.

### Measurements

- Memory utilisation;

- Process stability;

- Recovery time;

- Synchronisation behaviour;

- Manual intervention.

# 22. Experiment H - Combined Constraints

Combined experiments should be performed only after individual scenarios are understood.

Examples:

- Network interruption + CPU constraint;

- Network interruption + bandwidth limitation;

- Storage pressure + restart;

- Memory constraint + unexpected shutdown.

The purpose is to investigate whether multiple constraints create non-linear effects.

# 23. Recovery Definition

A key requirement of BNRB is a precise definition of “recovery.”

For the purposes of initial experiments, recovery may be defined as:

The point at which the node has returned to a stable operational state following disruption and has resumed its expected synchronisation and validation functions without unresolved errors.

The exact operational definition must be refined before data collection begins.

Different recovery states should be recorded.

### R0 - Failed

Node does not recover.

### R1 - Process Recovered

Bitcoin Core is running.

### R2 - Network Recovered

Node has re-established peer connectivity.

### R3 - Synchronisation Recovered

Node has returned to the expected chain state.

### R4 - Fully Operational

Node has completed recovery and is functioning normally.

This layered model may prove more useful than a single recovery-time metric.

# 24. Proposed Metrics

## Metric 1 - Time to Process Recovery

Time from restart to Bitcoin Core running.

## Metric 2 - Time to Network Recovery

Time from restart to stable peer connectivity.

## Metric 3 - Time to Synchronisation Recovery

Time until expected blockchain synchronisation state is restored.

## Metric 4 - Total Recovery Time

Time from disruption to full operational recovery.

## Metric 5 - Recovery Success Rate

Percentage of experiments resulting in successful recovery.

## Metric 6 - Manual Intervention Score

Amount of human intervention required.

## Metric 7 - Resource Recovery Profile

CPU, RAM, storage and network behaviour during recovery.

## Metric 8 - Data Integrity

Whether the node maintains valid and consistent blockchain data.

# 25. Data Collection

Each experiment must generate a standardised record.

The minimum dataset should include:

text id="w3g8ae" Experiment ID Date Time Node ID Hardware Profile OS Version Bitcoin Core Version Configuration Hash Baseline State Disruption Type Disruption Duration Start Time Recovery Start Process Recovery Network Recovery Sync Recovery Full Recovery CPU Usage Memory Usage Disk Usage Network Usage Manual Intervention Errors Observations Result

# 26. Experiment Metadata

Every experiment should have a unique identifier.

Example:

BNRB-2026-001
BNRB-2026-002
BNRB-2026-003

The metadata should allow independent researchers to reproduce the experiment.

# 27. Repetition

Experiments should not rely on a single trial.

Where practical, each experiment should be repeated.

The number of repetitions should be determined during pilot testing.

The objective is to distinguish:

- Random variation;

- Measurement error;

- Systematic behaviour.

# 28. Statistical Analysis

The initial analysis should focus on descriptive statistics.

For each experiment, calculate:

- Mean;

- Median;

- Minimum;

- Maximum;

- Standard deviation;

- Recovery success rate.

Where sample sizes permit, comparative analysis may be conducted between:

- Baseline and constrained nodes;

- Different hardware profiles;

- Different disruption types.

Statistical significance should not be claimed unless the experimental design supports such inference.

# 29. Reproducibility

The project should publish, where appropriate:

- Experimental configuration;

- Software versions;

- Hardware profiles;

- Experiment procedures;

- Measurement definitions;

- Analysis scripts;

- Non-sensitive datasets.

Each experiment should be documented sufficiently for independent reproduction.

# 30. Data Integrity

Research data should be stored in version-controlled repositories where practical.

Raw data should be preserved separately from processed data.

The project should maintain:

raw/
processed/
analysis/
reports/

Data transformations should be documented.

# 31. Experimental Safety

All experiments must be conducted in controlled environments.

The research must not:

- Attack public Bitcoin nodes;

- Conduct unauthorised network scanning;

- Attempt to disrupt the Bitcoin network;

- Target third-party infrastructure;

- Perform denial-of-service attacks;

- Test systems without authorisation.

Network disruption simulations must be limited to the research laboratory.

# 32. Power Testing

Power-related experiments must be conducted using controlled laboratory equipment.

Where possible, testing should use:

- Dedicated test hardware;

- Safe electrical equipment;

- UPS systems;

- Controlled power switching.

No experiment should create an electrical safety hazard.

# 33. Network Testing

Network disruptions must be simulated within the laboratory.

The experiment should use controlled network-management mechanisms to introduce:

- Delay;

- Packet loss;

- Bandwidth constraints;

- Connection interruption.

The public Internet should not be intentionally disrupted.

# 34. Storage Testing

Storage pressure must be simulated carefully.

Experiments should avoid uncontrolled filling of system partitions.

The research should use:

- Dedicated test storage;

- Controlled storage limits;

- Isolated experimental environments.

# 35. Security Safeguards

Experimental systems should:

- Use dedicated credentials;

- Avoid production wallets;

- Avoid storing significant funds;

- Be isolated from UBRO production systems;

- Use secure remote administration;

- Maintain access logs.

The research environment should be treated as a laboratory, not as a production Bitcoin financial system.

# 36. Ethical Considerations

The research will follow responsible research principles.

The project will:

- Minimise potential harm;

- Protect sensitive information;

- Avoid unnecessary collection of personal information;

- Respect third-party infrastructure;

- Clearly disclose limitations;

- Report negative findings.

# 37. Uganda Research Dimension

The initial laboratory will be based in Kampala.

The research may later investigate conditions relevant to other Ugandan locations.

Potential future sites include:

- Kampala;

- Jinja;

- Mbarara;

- Mbale.

However, geographic expansion will occur only after the core methodology is validated.

The research should not assume that a result observed in Kampala automatically applies to all of Uganda.

# 38. Research Phases

## Phase 1 - Protocol Review

Technical review of this protocol.

## Phase 2 - Laboratory Construction

Build and document the experimental environment.

## Phase 3 - Baseline Testing

Establish normal node behaviour.

## Phase 4 - Pilot Experiments

Conduct limited experiments.

## Phase 5 - Protocol Refinement

Adjust metrics and experimental procedures.

## Phase 6 - Main Experiments

Conduct the planned research.

## Phase 7 - Analysis

Analyse experimental results.

## Phase 8 - Publication

Publish findings and open-source materials where appropriate.

# 39. Proposed Timeline

## Month 1

- Technical review;

- Laboratory design;

- Hardware acquisition;

- Software setup.

## Month 2

- Baseline testing;

- Pilot experiments;

- Methodology refinement.

## Month 3

- Power experiments;

- Network experiments.

## Month 4

- Resource constraint experiments;

- Recovery experiments.

## Month 5

- Combined scenarios;

- Data analysis.

## Month 6

- Final analysis;

- Research paper;

- Open-source publication;

- Technical review.

# 40. Expected Outputs

At the end of the pilot, UBRO aims to produce:

### Output 1

BNRB Experimental Methodology.

### Output 2

BNRB Measurement Framework.

### Output 3

Experimental Dataset.

### Output 4

Analysis Tools.

### Output 5

Research Paper.

### Output 6

Open-Source Research Repository.

### Output 7

Technical Recommendations.

# 41. Success Criteria

The project will be considered successful if it:

- Establishes a reproducible experimental methodology;

- Successfully conducts controlled experiments;

- Produces reliable experimental data;

- Identifies measurable recovery behaviours;

- Clearly documents limitations;

- Produces useful open-source research artefacts;

- Receives independent technical feedback.

Success does not require proving the initial hypothesis.

A finding that the proposed resilience metrics are not useful would itself be a valid research result if properly documented.

# 42. Potential Contribution

If successful, BNRB could contribute to the Bitcoin ecosystem through:

- Research;

- Open-source tooling;

- Benchmarking methodology;

- Reproducible experiments;

- Technical documentation;

- Research datasets.

The project may also contribute to the broader understanding of decentralised infrastructure resilience.

# 43. Funding Principle

The research should follow a staged funding model.

### Stage 1

Discovery and methodology.

### Stage 2

Pilot laboratory.

### Stage 3

Six-month experimental programme.

### Stage 4

Extended research.

### Stage 5

Long-term research programme.

Funding should be requested based on demonstrated research requirements.

The project should avoid requesting large amounts before establishing the technical case.

# 44. Technical Review Requirement

Before the protocol is implemented, UBRO should seek independent technical review.

At minimum, feedback should be requested on:

- Research question;

- Experimental design;

- Metrics;

- Recovery definitions;

- Reproducibility;

- Potential duplication;

- Research ethics.

The protocol should be revised based on credible technical feedback.

# 45. Final Position

BNRB is not proposed as a replacement for Bitcoin Core testing, Bitcoin network monitoring or existing node-management platforms.

The proposed research occupies a different potential space:

Experimental measurement of operational resilience and recovery behaviour at the individual Bitcoin full-node level.

Whether this represents a sufficiently important and novel research contribution remains to be determined.

The purpose of the BNSAT-R programme is to establish that through evidence.

# 46. Final Research Principle

Do not build because we believe there is a gap.

Research whether the gap exists.

Measure the problem.

Validate the methodology.

Then build only if the evidence justifies it.

UBRO CLASSIFIEDS LIMITED

BNSAT-R - Bitcoin Node Security & Resilience Research

Research. Measure. Build. Contribute.